1. Data Acquisition Layer (Python Backend)

- LSL Stream Listener: Connects to the Muse EEG stream using pylsl.

- Ring Buffer: Stores a rolling 2-second window of raw voltage data.

- Preprocessing Pipeline: * Notch Filter: Removes 60Hz electrical hum.

  - Bandpass Filter: Isolates 1Hz–40Hz (removes "drift" and muscle noise).

2. Signal Processing Engine (Python Backend)

- FFT Module: Uses scipy.signal.welch to calculate power across all frequencies.

- Band Extractor: Pulls specific values for Delta (1-4Hz), Theta (4-8Hz), Alpha (8-13Hz), Beta (13-30Hz), and Gamma (30-40Hz).

- Ratio Logic: Computes the user-selected formula (e.g., $Alpha / Beta$).

- Threshold Manager: Compares live ratio to the Baseline (stored during calibration).

3. Neurofeedback Audio Logic (Python Backend)

- Indicator Engine (pygame.mixer):

  - Loops rain.wav.

  - Proportional Logic: Volume = $1.0 - (\text{SuccessRatio})$. (As you improve, rain fades).

- Reward Engine:

  - Randomly selects a .wav from the /birds/ folder.

  - Triggers playback when ratio exceeds the threshold for $>0.5$ seconds.

4. The Frontend (C# WPF App)

- Tab 1: Monitoring (Signal Quality)

  - Raw EEG Plot: 4 labeled lines (AF7, AF8, TP9, TP10).

  - Signal Integrity Lights: Red/Yellow/Green indicators based on sensor contact.

  - Live Power Bar Graph: Real-time dancing bars for each band.

- Tab 2: Neurofeedback (Session Control)

  - Ratio Dropdowns: Selector for Numerator (Alpha/Beta/etc) and Denominator.

  - Calibration Button: Triggers the 60s baseline recording.

  - Threshold Slider: Manual override for the bird-trigger sensitivity.

- Tab 3: Participant & Study Manager

  - Metadata Entry: Fields for Participant ID, Session Number, and Age.

  - LabRecorder Control: "Start Recording" button that triggers the TCP command to LabRecorder.

  - Memory Test Interface: Integrated window to display words/numbers for the participant to recall.

5. Communication Bridge (IPC)

- Local Server: A WebSocket server (Python) that broadcasts a JSON object every 100ms.

  - Payload: { "alpha": 0.5, "beta": 0.2, "ratio": 2.5, "waves": [...] }

- C# Client: Receives the JSON and updates the UI elements.