April 29, 2026 Dev Journal

Daily Summary

The project underwent a significant architectural shift today. We moved from a non-functional Windows 10 Bluetooth environment to a functional Windows 11 setup, identified critical signal sensitivity hurdles, and established the foundation for a Python-to-C# WebSocket bridge.

Progress & Achievements

1. Hardware & Environment Pivot

- The Problem: The Windows 10 development rig failed to detect the Muse 2.

- The Diagnosis: Checked the Link Manager Protocol (LMP) in Device Manager; found version 3.3164 (Bluetooth 2.0), which lacks the BLE support required for EEG streaming.

- The Fix: Transitioned to a Windows 11 machine with native BLE hardware, successfully establishing the LSL stream.

2. Signal Processing (DSP) Refinement

- Artifact Discovery: Initial testing revealed extreme sensitivity to muscle movement (EMG). The loop was effectively "hanging" because it was rejecting almost every chunk due to eye blinks and jaw clenches.

- Filter Logic: Narrowed the Bandpass filter from 1-40Hz down to 1-30Hz to strip out more high-frequency noise.

- Threshold Adjustment: Bumped the artifact gating from 100uV to 150uV to allow for micro-movements while still catching major artifacts.

- State Management: Added a "Hold" mechanism (last_valid_metrics) so the system maintains the last good reading during a blink instead of crashing the data flow.

3. Bridge Architecture (The "Nervous System")

- Backend: Rewrote engine.py as a lightweight WebSocket server broadcasting on port 8765.

- FFT Implementation: Integrated scipy.signal.welch to calculate Alpha (8-12Hz) and Beta (13-30Hz) powers, outputting as JSON payloads.

- Frontend: Initialized the C# WPF project with ScottPlot.WPF for real-time EEG and PSD (Power Spectral Density) graphing.

Problems & Technical Hurdles

- Dependency Error: Encountered a metadata generation failure when trying to pip install lsl. Resolved by switching to the correct library: pylsl.

- Performance Constraints: Confirmed that matplotlib is too resource-heavy for a 256Hz real-time stream, necessitating the move to a C# visualization layer.

Next Objectives (For April 30)

- Run the Full Stack: Connect the C# WebSocket client to the Python server and verify the data plots.

- Calibration Phase: Implement the 60-second baseline recording to normalize the Alpha/Beta ratios for different participants.