April 30, 2026 Dev Journal

Daily Summary

Today focused on elevating the "Worker" (Python) and "General" (C#) into a cohesive research suite. While the math and audio logic are now high-end, we have identified a significant "blind spot" in the data pipeline where information is calculated but not successfully rendered or broadcasted to external tools.

Progress & Achievements

1. Signal Stability (The 3-Second "Glide")

- Math Upgrade: Implemented an Exponential Moving Average (EMA) with a factor of 0.1 on the PSD array.

- Significance: This creates the industry-standard 3-second smoothing window, eliminating the jitter that makes neurofeedback sound "staccato" or distracting.

2. C# Audio & UI Overhaul

- Audio Engine: Migrated all sound logic to C# using NAudio to eliminate WebSocket latency. Volume changes now use linear interpolation (50ms) to ensure smooth transitions without audio pops.

- UI Organization: Redesigned the frontend into a 5-tab "Control Center" (Signals, Trends, Neurofeedback, Settings, External Tools).

- Process Management: The Python engine now runs as a hidden background process with its output piped directly into an integrated terminal in the WPF app.

3. Hardware Success

- Successfully integrated a Bluetooth 5.0 dongle, resolving the BLE compatibility issues from yesterday and ensuring a stable connection to the Muse 2/S.

Hardships & Persistent Challenges (The "Unsolved" List)

1. The LSL "Invisibility" Bug

- The Issue: Although the Python engine calculates metrics perfectly, the LSL metrics stream is not appearing in LabRecorder.

- Attempts: Forced a nominal sampling rate of 10.24Hz and moved Outlets to global scope, but the stream remains invisible to external recording tools.

2. Frontend Data Binding Failures

- Graphing: The 2x2 sensor grid (TP9, TP10, AF7, AF8) remains blank despite mean-centering implementation. ScottPlot DataStreamers are failing to render incoming arrays.

- UI Readouts: Delta, Theta, and Gamma indicators are stuck at 0.00.

- Probable Cause: Likely a JSON property naming mismatch between the Python dictionary keys and C# deserialization, or a race condition during ScottPlot initialization.

Current Project State

- Bluetooth/LSL Acquisition: [Working].

- DSP Math (EMA/FFT): [Working/Clinical-Grade].

- Audio Feedback: [Working/Smooth].

- Data Visualization: [CRITICAL FAILURE - Debugging Required].